Kaypro 10 Emulation Implementation Plan (Complete, Phased)

Summary

Add a new Kaypro 10 machine configuration to the Rust emulator with:

- WD1002-05 hard disk controller emulation (ports 0x80-0x87)

- Virtual ST412-style hard disk image support

- One physical DSDD floppy device

- BIOS-driven logical drive assignment (A/B/C) with no emulator hard-coding

- Phase-1 boot-trace validation requiring A0> prompt detection

This plan is incremental and test-gated so each major subsystem is verifiable before advancing.

Locked Decisions and Defaults

1) Model naming

- User-visible model: Kaypro 10

- CLI selector: --model kaypro10

- Config/serde value: kaypro10

- Internal enum variant: Kaypro10

2) Floppy default image

- Default Kaypro10 floppy image: disks/system/k10u-rld.img

3) Hard disk image format

- Java-compatible WD1002 image format with trailing 128-byte geometry header

- Default geometry: 306c4h512z17p1l

- Default total file size: 10,653,824 bytes

4) Phase-1 boot validation

- Must detect strict A0> prompt (not A> fallback)

- Add Kaypro10-specific boot trace diagnostics for failure triage

5) Drive letters

- Emulator models physical devices only

- ROM BIOS determines logical mapping:

- Unformatted HD: floppy may be booted as A:

- Post HDFMT + PUTSYSU: HD partitions A:/B:, floppy C:

Public API and Interface Changes

1) src/config.rs

- Add KayproModel::Kaypro10 with serde kaypro10

- Add Kaypro10 defaults:

- ROM: roms/81-478c.rom

- Floppy: disks/system/k10u-rld.img

- HD image path default (new field): e.g. disks/system/WD1002-1

- Add optional config field for hard disk image path (e.g. hard_disk)

2) src/main.rs

- Extend model help text/list with kaypro10

- Add optional CLI override for hard disk image (e.g. --hd <FILE>)

- Keep existing legacy model behavior unchanged

3) src/kaypro_machine.rs

- Add model-aware device routing for Kaypro10

- Route IN/OUT 0x80-0x87 to WD1002 in Kaypro10 mode

- Keep current behavior for existing models (no regression)

4) New modules

- src/wd1002.rs: controller/task-file/status/command engine

- src/hard_disk_image.rs: file-backed geometry-aware storage with header support

5) src/diagnostics.rs

- Add Kaypro10 entry to boot-test matrix

- Add boot trace routine for Kaypro10 Phase-1 validation

- Add strict A0> signature checking mode

Architecture and Data Flow

1) CPU executes ROM BIOS (81-478c)

2) BIOS probes devices via system/FDC/HDC I/O ports

3) KayproMachine routes:

- FDC ports 0x10-0x13 to floppy controller

- HDC ports 0x80-0x87 to WD1002 (Kaypro10 only)

4) WD1002 reads/writes sectors from hard_disk_image backend

5) BIOS boot path:

- Attempts HD boot first when probe indicates available

- Falls back to floppy if HD boot-sector validation fails

- After formatting/system install, boots from HD

Phased Implementation

Phase 1: Kaypro10 Profile + Boot Trace Gate

Scope:

- Add kaypro10 model/config plumbing

- Set default floppy to disks/system/k10u-rld.img

- Add Kaypro10 boot test entry

- Add Kaypro10 boot tracing routine and strict A0> detection gate

Acceptance:

- --boot-test includes Boot Kaypro 10

- Kaypro10 boot test reports pass only when A0> is observed

- On failure, diagnostics include trace summary + screen snapshot + PC state

Phase 2: Hard Disk Image Backend

Scope:

- Implement open/create/read/write with geometry checks

- Implement trailing 128-byte header parse/write

- Support mounting doc/kaypro10/WD1002-1 reference image

Acceptance:

- Unit tests validate:

- file size (10,653,824)

- geometry/header roundtrip

- sector offset correctness and bounds errors

Phase 3: WD1002 Register and Status Core

Scope:

- Implement task-file registers (8 ports)

- Status/error bit model

- Reset/diagnostic behavior compatible with ROM/HDFMT expectations

- LUN handling (LUN 1 behavior)

Acceptance:

- Unit tests for reset state, register access, LUN readiness/error handling

Phase 4: WD1002 Command Execution

Scope:

- Implement required commands:

- RESTORE (0x10)

- READ (0x20)

- WRITE (0x30)

- FORMAT TRACK (0x50)

- SEEK (0x70)

- Implement DRQ/BUSY/error sequencing and data transfer loops

Acceptance:

- Unit tests for command lifecycle and persistent data integrity

- Format-track command accepted by expected utility flow

Phase 5: Machine Integration (Kaypro10 I/O)

Scope:

- Integrate WD1002 into KayproMachine constructor/state

- Port decode for 0x80-0x87 in Kaypro10 mode

- Preserve existing models exactly

Acceptance:

- ROM probe path reaches HDC routines without hangs

- Existing model boot tests remain green

Phase 6: First-Boot Behavior Validation (Blank HD)

Scope:

- Validate unformatted HD behavior

- Ensure no emulator-level forced letter mapping

Acceptance:

- BIOS attempts HD boot then falls back to floppy boot path

- Boot behavior aligns with real Kaypro10 expectations

Phase 7: HDFMT Validation

Scope:

- Run HDFMT in emulator against WD1002/virtual HD

Acceptance:

- HDFMT completes without controller-level failure

- Disk image mutates consistently with low-level format operations

Phase 8: PUTSYSU Validation + HD Boot Transition

Scope:

- Run PUTSYSU after format

- Reboot and verify HD boot success

Acceptance:

- System boots from HD post-install

- Runtime logical mapping follows BIOS behavior (A/B HD, C floppy)

Phase 9: Regression, Diagnostics, Documentation

Scope:

- Add/expand tests and docs for Kaypro10 setup/flows

- Document default floppy image and HD creation behavior

- Document first-boot/unformatted-HD fallback behavior

Acceptance:

- All prior model tests pass

- Kaypro10 smoke/boot tests pass consistently

- README/config docs updated with Kaypro10 instructions

Test Cases and Scenarios

1) Boot-trace positive: Kaypro10 reaches strict A0>

2) Boot-trace negative: wrong prompt (A>/B0>) fails strict gate

3) Boot-trace negative: timeout with trace context

4) WD1002 reset/register conformance

5) WD1002 read/write/format command behavior

6) Blank HD boot fallback to floppy

7) HDFMT successful execution

8) PUTSYSU successful execution and post-install HD boot

9) Non-Kaypro10 model regression suite

Risks and Mitigations

1) Prompt variability across ROM/disk combinations

- Mitigation: strict A0> gate only for selected Phase-1 fixture; include rich trace output for quick diagnosis

2) Timing-sensitive controller polling loops

- Mitigation: implement deterministic state transitions matching ROM/HDFMT expectations; add command-level tests

3) Hidden regressions in existing models

- Mitigation: preserve legacy code paths and run full boot-test matrix every phase

Assumptions

- disks/system/k10u-rld.img is the canonical default floppy for Kaypro10 boot validation

- Phase-1 fixture is expected to produce A0> when machine is correctly bootable

- Initial WD1002 feature scope is limited to ROM + HDFMT + PUTSYSU requirements